POV INSPECTION CHECKLIST

| ITEM WHAT TO CHECK |  | LOOK FOR KNOWN DEFICIENCIES | CHECK OFF |
| --- | --- | --- | --- |
| TIRES Condition NOTE: No mixing of radial tires and bias tires. Snow Tire Auth 15 Nov – 1 Apr Spare tire | Tread depth, wear, weathering, evenly seated, bulges, imbedded objects, cuts, breaks. At least one mm of tread over entire traction surface. (Using a penny, place it in the tire tread with head facing downward. If the tread does not reach the top of Lincoln's head, there is insufficient tread depth) Spare tire (inflated), jack, lug wrench |  | Front Rear Pass Fail |
| LIGHTS Head lights Tail Lights Brake lights Turn Signals Backup lights Four-way Flashers License Plate Light | Both high and low beams operational, cracked, condensation, secured Lenses intact, tail light working when turned on (red) Lenses intact, brake light working when brake is applied (red) Lenses intact, left and right turn signals blink (red lights in rear and yellow lights in front) Lenses intact, left and right backup lights work (White Light) Lenses intact, left and right turn signals flash/blink at the same time Lenses intact, does light stay on |  | Left Right Left Right Left Right Front Rear Left Right Left Right Front Rear Left Right Pass Fail |
| WINDSHIELD & WINDOWS & WIPERS Windshield Rear Window Windows Window controls Windshield wipers | Not cracked, broken or scratched to the degree that impairs vision Not cracked, broken or scratched to the degree that impairs vision Windows go up and down, scratched or tinted to the degree that impairs vision Check handles, push electric buttons Both wipers are installed on vehicle, windshield wipers work, blades show signs of wear |  | Pass Fail Pass Fail Pass Fail Front Rear Pass Fail |
| MIRROR Mirror Outside Mirror Inside | Missing, cracked Missing, cracked |  | Left Right Pass Fail |
| BUMPERS Bumper Front Bumper Rear | Missing, loose, broken Missing, loose, broken, bent in any way to cause a hazard |  | Pass Fail Pass Fail |
| BRAKES Brakes Emergency Brake | Foot pedal cannot travel more than half way to floor, does brake light stay on Properly adjusted, check emergency brake by: pull/push emergency brake, apply foot to brake, gently press gas pedal, ensure brake holds vehicle |  | Pass Fail Pass Fail |
| Interior Horn Defroster Front Defroster Rear Emergency equipment Heater SEATBELTS Seatbelt Front/Rear (Include shoulder harness during inspection, may have a center seat belt) | Does it work Ensure hot air blows out above the dash Check light on dash, if in the winter ensure it works by allowing the rear windshield to clear up (OPTIONAL) First aid kit, warning triangle, flashlight, fire extinguisher, blanket, flares, shovel, chains, tools, etc. (Check host nation laws for any additional equipment) Ensure heater works Missing, frayed, does not snap |  | Pass Fail Pass Fail Pass Fail Pass Fail Pass Fail Front Rear |
| LICENSE/DECALS/INSURANCE State Drivers License Installation decal License Plate/ Inspection Decal Insurance | Expired, missing Missing, needs replacing Expired, check sticker/decal to ensure plate is current Does the operator have valid insurance |  | Pass Fail Pass Fail Pass Fail Pass Fail |
| UNDER THE HOOD FLUIDS Brake Windshield washer Battery Power Steering HOSES BATTERY | Filled to appropriate level Windshield washer fluid Check the color indicator on the battery Filled to appropriate level Cuts, cracks, leaks, bulges, chaffing, deterioration Terminals, clean and tight, held down securely |  | Pass Fail Pass Fail Pass Fail Pass Fail Pass Fail Pass Fail |

Inspector's Name:_________________________Signature__________________________

Operator Name:_________________________Signature__________________________

Platoon Sergeant/Platoon Leaders approval_________________________________________

Date inspection was conducted_________

Date follow-up inspection was conducted__________

Leave/Pass/Holiday________________

Inspection checklist can be revised based on local requirements - e.g., snow tires/chain